Activity 3: Calibration practical: Teacher and technician notes

Station 1 – Calibrating a balance

Equipment needed per student

- A mass balance, minimum of 2 d.p but 4 d.p is preferred

- A set of masses that can be used as calibration masses

You may have to add into the worksheet (or demonstrate to students) how to calibrate the pieces of equipment used.

Safety

Ensure risk assessments have been conducted and checked by an appropriate practitioner.

Station 2 – Calibrating a pH meter

Equipment

- pH meter

- pH meter calibration buffer solutions (pH 4.0, pH 7.0, pH 10.0) in small beakers

- Distilled or deionised water

- Lint-free tissue

- Beakers for rinsing

Safety

If using tablets or commercial solutions, refer to any hazard warning labels on the containers.

If using prepared buffers (from CLEAPSS recipes below) they are currently not classified as hazardous.

- It is recommended (by CLEAPSS) to use a standard buffer solution prepared from tablets or commercial solutions to calibrate pH meters.

- If buffer solutions need to be prepared, then instructions can be found at: science.cleapss.org.uk/Resource/RB018-Buffer-solutions.pdf

Station 3 – Calibrating a pipette

Equipment per student

- Mechanical (variable volume) pipette

- Calibrated high-sensitivity balance (or at least 0.0001 g)

- Weighing boats or small containers

- Distilled or deionised water in clean flasks/beakers

- Thermometer

Safety

Ensure risk assessments have been conducted and checked by an appropriate practitioner.

Station 4 – Calibrating a conductivity meter

Equipment per student

- Conductivity meter

- Standard conductivity calibration solution(s) (150 µS/cm conductivity standard, 1413 µS/cm conductivity standard and 2880 µS/cm conductivity standard)

- Distilled or deionised water

- Clean beaker(s) for the standard solution(s)

- Lint-free tissue

Safety

Refer to solution manufacturer’s safety instructions. Ensure risk assessments have been conducted and checked by an appropriate practitioner.

A video for how to prepare some calibration solutions can be found below: www.youtube.com/watch?v=efEdmZfRRrs