Worksheet

Introduction

Fill in the labels on the diagram to complete the reflux setup.

Activity 1 – Reflux demonstration

Task 1: Explain briefly how reflux works. Include a description of the state changes.

Task 2:

- What is the definition of oxidation?

- Write the word and symbol equation for the oxidation of ethanol into ethanoic acid and water. (Use [O] to show the oxidising agent.)

- How can we verify that ethanoic acid was made in the reaction?

Activity 2 – Reflux in industry

Watch the video (https://vimeo.com/1060452181) and identify three different uses of reflux in industry.

Do some quick research to explain how the use of reflux is beneficial in each case.

Activity 3 – Troubleshooting reflux

Consider the reported problems/observations listed below relating to a reaction under reflux and suggest solutions or explanations for what is going wrong.

| Reported problem/observation from chemists | Suggested solution |
| --- | --- |
| “I am not observing any drips falling from the condenser. Boiling is weak/not occurring. The equipment is heated with a stirrer hotplate set to 70°C and the solvent is ethanol.” |  |
| “The condenser is getting hot and vapour is escaping from the open (top) end.” |  |
| “The condenser is attached to the water and the water is on, but the condenser is not filling correctly, and there are a lot of bubbles and air space in the water jacket.” |  |
| “The boiling liquid is bubbling into the condenser and leaking from the joint between the flask and the condenser. I am using a 100 ml flask, and there is around 85 ml of liquid total in the reaction.” |  |

Plenary

- State when a refluxing technique might be used during organic synthesis.

- Give the definition for oxidation in terms of loss or gain of oxygen, hydrogen, and electrons.

- In terms of loss or gain of oxygen and the loss or gain of hydrogen, explain whether the reaction to form hexanoic acid from hexanol is an oxidation or a reduction.

- Write the word equation and the symbol equation for the oxidation of butan-1-ol into a carboxylic acid.

- Describe the purpose and positioning of the condenser in a basic laboratory reflux setup, including the water flow in and out.

- Explain why the drip rate from the condenser back into the flask below is important when performing a reflux.

- Explain the use of anti-bumping granules in a reflux setup.

- Explain why the position of the condenser and the open system are important for high yields and safety within a reflux.