Activity 3 Worksheet: Calculating unit rates

Calculate the three different unit rates included in the extracts from the Bill of Quantities (BoQ) shown below. 
In industry, the calculation of unit rates is known as building-up unit rates.

| Item | Description | Quantity | Unit | Unit rate | Total |
| --- | --- | --- | --- | --- | --- |
| a | Foundation excavation not exceeding 2 m deep. |  | m3 |  |  |
| b | A142 mesh reinforcement 2.22 kg per m2, minimum 300 mm side and end laps. |  | m2 |  |  |
| c | One brick wall in type B facings, in 1:1:6 mortar, pointed both sides as the work proceeds. |  | m2 |  |  |

Item a

Using the information provided on the Activity 3 Reference sheet calculate the unit rate for:

Foundation excavation not exceeding 2 m deep.

(Hint: You will need an excavator, an excavator driver and a banksman)

Item b

Using the information provided on the Activity 3 Reference sheet calculate the unit rate for:

A142 mesh reinforcement 2.22 kg per m2, minimum 300 mm side and end laps.

(Hint: You will need A142 mesh, spacer block/chairs, tie wire and a steel fixer)

Item c

Using the information provided on the Activity 3 Reference sheet calculate the unit rate for:

One brick wall in type B facings, in 1:1:6 mortar, pointed both sides as the work proceeds.

(Hint: You will need bricks, mortar, a bricklayer and a labourer)