Calculating heat loss through a wall

# Worked example

A typical cavity wall consists of two layers (or leaves) of masonry with a gap (cavity) between them. The materials used in each layer and the cavity itself have different thermal resistances.

Image © Shutterstock/Irene Miller

The common makeup of a cavity wall consists of:

| Material | Thermal resistance, R, (in m2 K/W) |
| --- | --- |
| 100 mm clay brick | 0.18 |
| 50 mm air gap | 0.18 |
| 50 mm expanded polystyrene board | 1.25 |
| 100 mm concrete block | 0.15 |
| Outside surface resistance | 0.06 |
| Inside surface resistance | 0.12 |

U-value

U=1R

where R is the thermal resistance

Calculate the heat loss by 1 m2 of this wall when the temperature difference either side of the wall is 10°C.

For this cavity wall, the thermal resistance is

R = 0.18 + 0.18 + 1.25 + 0.15 + 0.06 + 0.12 = 1.94

U-value for the cavity wall:

U= 1R= 11.94=0.515 W/m2K

## Rate of heat loss

Heat loss:

Q=U A ΔT

The temperature difference from one side of the cavity wall to the other was 10°C, or 10 K.  The heat loss for 1 m2 of this cavity wall would be:

Q =U A ΔT

Q=0.515×1×10

Q=5.15 Watts