Minimising heat loss using differential calculus

# Worked example

A box-shaped house has a square floor. Three times as much heat per square metre is lost through the roof as through the walls, but no heat is lost through the floor.

If the house encloses 1500 m3, find the area of the floor that minimises heat loss and the height of the house.

Remember: The house is a cuboid. Its volume can be calculated by base × width × height.

Step 1: Define the variables.

- Let w be the length of the base of the square floor in metres.

- Let h be the height of the house in metres.

The volume of the house (V) is 1500 m3, so:

V= w2h=1500

Solve for h:

h=1500w2

Step 2: Set up the heat loss equations.

Q=UA ΔT

The total heat loss comes from two components:

- heat lost through the walls;

- heat lost through the roof.

Heat loss through the walls

The house has four vertical walls. Each wall has an area of wh (base × height). 
Therefore, the total wall area (A) in terms of w is:

A=4(wh)

A=4w 1500w2

A=4 1500w

A=6000w

We know that the heat loss through the walls is proportional to the wall area. We can use the heat loss equation for the wall:

Q=UA ΔT

Qwalls= Uwalls 6000wΔT

Heat loss through the roof

The area of the roof is the same as the area of the floor.

The area of floor = w2 (base × base)

We are told that three times as much heat per square metre is lost through the roof as through the walls. This means we can find the U-value for the roof:

Uroof=3Uwalls

Therefore, the heat loss through the roof is:

Q=UA ΔT

Qroof= 3Uwalls w2ΔT

Step 3: Combine the heat loss equations to calculate total heat loss.

The total heat loss, Q, is the sum of the heat loss through the walls and the roof:

Q= Qwalls +Qroof

Q= Uwalls 6000wΔT+3Uwalls w2ΔT

Take out the common factors of Uwalls and  ∆T from both terms:

Q= Uwalls ΔT 6000w+3w2

Step 4: Use differentiation to find the point at which heat loss is at a minimum.

- To find the length of one side of the floor (w) which minimises heat loss, we need to find the critical points.

- We need to differentiate the total heat loss function, Q, with respect to w and then set the derivative equal to zero.

We can now ignore the constant factor Uwalls ΔT as this will not affect the minimisation.

Rewrite the function as:

q(w)= 6000w+3w2

First, rewrite the equation using a negative index.

q(w)= 6000w-1+3w2

Now differentiate q with respect to w.

Remember: When differentiating, we multiply the coefficient of w by the index and then reduce the index by 1. For example: 4w3 becomes 12w2

dqdw= -6000w-2+6w

dqdw= -6000w2+6w

Step 5: Set the derivative equal to zero to find the critical points, and solve the equation.

At the critical points dqdw=0, so:

-6000w2+6w=0

Now, solve the equation. Start by rearranging to make any negative terms positive.

-6000w2+6w=0

6w=6000w2

6w3=6000

w3=60006=1000

w =31000=10 m

Step 6: Find the height.

Now that we know w=10 m, we can find the height h using the volume equation:

h=1500w2

h=1500102

h=1500100=15 m

Step 7: Verify it’s a minimum.

To verify that this is a minimum value, we can check the second derivative of q.

Remember: If the second derivative is negative, the critical point will be a maximum. 
If the second derivative is positive, the critical point will be a minimum.

To find the second derivative, differentiate the first derivative.

dqdw= -6000w-2+6w

Remember: When differentiating w (which is equivalent to w1), the index reduces to 0 to give w0. Anything to the power of 0 is equal to 1, so the derivative of w is 1.

d2 Qdw2= -2× -6000w-3+6

d2 Qdw2= 12 000w-3+6

d2 Qdw2= 12 000w3+6

Substitute in the value of w, w=10

12 000103+6= 12 0001000+6=12+6=18

Since 18 > 0, the second derivative is positive, confirming that w=10 gives a minimum.

Final answer

The area of the floor that minimises heat loss is 10 × 10 = 100 m2 and the height of the house is 15 metres.