Activity 2: Worksheet answers

# Practice question 1

Step 1: Plot data as shown.

Step 2: Divide the base width into equal intervals

Interval width = 10 ÷ 5 = 2 m

Steps 3 and 4: Interval midpoint heights

Interval 1 = 1.5 m 		Interval 2 = 2.5 m

Interval 3 = 3 m 		Interval 4 = 2.5 m

Interval 5 = 1.5 m

Step 5: Interval areas

Interval 1 area = 1.5 m × 2 m = 3 m2		Interval 2 area = 2.5 m x 2 m = 5 m2

Interval 3 area = 3 m × 2 m = 6 m2 		Interval 4 area = 2.5 m × 2 m = 5 m2

Interval 5 area = 1.5 m × 2 m = 3 m2

Step 6:

Area of fill = 3 + 5 + 6 + 5 + 3 = 22 m2

Step 7:

Total volume of fill material required = 22 × 15 = 330 m3

# Practice question 2

Step 1: Sketch the embankment onto graph paper using the values in the table.

| Position (m) | Proposed height (m) |
| --- | --- |
| 0 | 2.5 |
| 2 | 3 |
| 4 | 5 |
| 6 | 5 |
| 8 | 5 |
| 10 | 3 |
| 12 | 2.5 |

Step 2: Divide the base width into equal intervals.

The base width is 12 m. The number of intervals is 6.

Therefore, the interval width = 12 ÷ 6 = 2 m

Steps 3 and 4: Find the midpoint heights of each interval.

Interval 1 = 2.75 m 		Interval 2 = 4 m

Interval 3 = 5 m 			Interval 4 = 5 m

Interval 5 = 4 m			Interval 6 = 2.75 m

Step 5: Calculate the area of each interval.

Interval 1 area = 2.75 m × 2 m = 5.5 m2 		Interval 2 area = 4 m × 2 m = 8 m2

Interval 3 area = 5 m × 2 m = 10 m2 		Interval 4 area = 5 m × 2 m = 10 m2

Interval 5 area = 4 m × 2 m = 8 m2 		Interval 6 area = 2.75 m × 2 m = 5.5 m2

Step 6: Add up the areas of the intervals to get the total area of the cross-section.

5.5 + 8 + 10 + 10 + 8+ 5.5 = 47 m2

# Practice question 3

Step 1: Sketch the road onto graph paper using the values in the table.

Step 2: Divide the base width into equal intervals.

The base width is 10 m. The number of intervals is 5.

Therefore, the interval width is 10 ÷ 5 = 2 m.

Steps 3 and 4: Find the midpoint heights of each interval.

Interval 1 = 0.25 m 		Interval 2 = 0.25 m

Interval 3 = 0.5 m 			Interval 4 = 0 m

Interval 5 = –0.75 m

Step 5: Calculate the area of each interval.

Interval 1 area = 0.25 m × 2 m = 0.5 m2 		Interval 2 area = 0.25 m × 2 m = 0.5 m2

Interval 3 area = 0.5 m × 2 m = 1 m2 		Interval 4 area = 0 m × 2 m = 0 m2

Interval 5 area = –0.75 m × 2 m = –1.5 m2

Step 6: Add up the areas of the intervals to get the total area of the cross-section.

0.5 + 0.5 + 1 + 0 – 1.5 = 0.5 m2

Step 7: Calculate the volume of earth needing to be removed.

0.5 × 9 = 4.5 m3