Activity 1: Worksheet answers

# Practice question 1

Step 1: Plot the data.

This was done for you.

Step 2: Divide the base width into equal intervals.

The base width is 10 m. The number of intervals is 5.

Therefore, the interval width is 10 ÷ 5 = 2 m.

(See graph below.)

Step 3: Identify the ordinates on the graph

Step 4: Find the heights of the ordinates.

y1=1 m

y2=2 m

y3=3 m

y4=3 m

y5=2 m

y6=1 m

Step 5: Use the trapezoidal rule to calculate the area of cut and fill.

Area ≈ 12 interval width ×first ordinate+2(sum of middle ordinates)+last ordinate

interval width = 2

first ordinate = 1

sum of middle ordinates = 2 + 3 + 3 + 2 = 10

last ordinate = 1

Area ≈ 22 ×[1+210+1]

Area ≈  1 ×1+210+1

Area≈  1+20+1

Area≈ 22 m2

Step 6: Calculate the total volume of fill material required.

Total volume of fill material required = 22 × 15 = 330 m3

# Practice question 2

Step 1: Plot the embankment onto graph paper using the values in the table.

Step 2: Divide the base width into equal intervals.

The base width is 12 m. The number of intervals is 6.

Therefore, the interval width is = 12 ÷ 6 = 2 m

Step 3: Identify the ordinates on the graph.

These should be labelled as y1 to y7.

Step 4: Find the heights of the ordinates:

y1=2.5 m

y2=3 m

y3=5 m

y4=5 m

y5=5 m

y6=3 m

y7=2.5 m

Step 5: Use the trapezoidal rule to calculate the area of cut and fill.

Area ≈ 12 interval width ×first ordinate+2(sum of middle ordinates)+last ordinate

first ordinate = 2.5

sum of middle ordinates = 3 + 5 + 5 + 5 + 3 = 21

last ordinate = 2.5

Area ≈ 22 ×[2.5+221+2.5]

Area ≈ 1 ×[2.5+221+2.5]

Area ≈ 2.5+42+2.5=47 m2

Area ≈ 47 m2

# Practice question 3

Step 1: Plot the embankment onto graph paper using the values in the table.

Step 2: Divide the base width into equal intervals.

The base width is 10 m. The number of intervals is 8.

Therefore, the interval width is 16 ÷ 8 = 2 m.

Step 3: Identify the ordinates on the graph

These should be labelled as y1 to y9.

Step 4: Find the heights of the ordinates.

The areas of cut are positive values as they are above the proposed new level.

y1=0.5 m

y2=1 m

y3=0.5 m

y4=0.25 m

y5=0 m

The areas of fill are negative values as they are below the proposed new level.

y6=-1 m

y7=-1.5 m

y8=-2 m

y9=-2 m

Step 5: Use the trapezoidal rule to calculate the area of cut and fill.

Area ≈ 12 interval width ×first ordinate+2(sum of middle ordinates)+last ordinate

first ordinate = 0.5

sum of middle ordinates = 1 + 0.5 + 0.25 + 0 – 1 – 1.5 – 2 = –2.75

last ordinate = –2

Area ≈ 22 ×0.5+2-2.75-2

Area ≈ 1 ×0.5+2-2.75-2

Area ≈ 0.5-5.5-2

Area ≈ -7 m2

The result is negative, which means that more material is needed than there is on the site. The earthworks contractor will need to bring 7.0 m2 of earth per metre width of road (7.0 m3) to the site.

Step 6: Calculate the total amount of soil to be added.

7 × 9 = 63 m3