Box optimisation answers

# Worked example 1 – Box size optimisation

[Image © Shutterstock/Richard Peterson]

Basic equation 		Component dimensions

V = L × W × H		Length (L) =10-2x

Width (W) =5-2x

Height (H) =x

The primary goal of this problem is not to solve for the maximum volume, but to practise the algebraic skills (expansion) and the differentiation required:

- Derive the volume function.

V=  L × W × H

L=10-2x        W=5-2x          H=x

V=(10-2x)(5-2x)(x)

This gives the volume in terms of x.

- Expand the expression to form a cubic polynomial.

V=(10-2x)(5-2x)(x)

Multiply 10 by everything in the second brackets.

50-20x

Multiply -2x by everything in the second brackets.

-10x+4x2

Then combine the results

V = (50-20x-10x+4x2)(x)

and simplify

V = (4x2-30x+50)(x)

Continue expanding the bracket.

V = (4x2-30x+50)(x)

Multiply all terms in the first brackets by x.

V = 4x3-30x2+50x

So, the volume of the box is represented by the polynomial

V =4x3-30x2+50x

- Find the derivative dVdx.

V =4x3-30x2+50x

When differentiating polynomials, multiply the coefficient by the index, then reduce the index by one.

dVdx =ddx4x3-30x2+50x

=3×4x2-2×30x+50

=12x2-60x+50

# Worked example 2 – Finding the optimal point

Problem

The cost (C) of running a specific manufacturing process is given by:

C(s)=2s2-24s+150, where s is the operating speed.

Find the operating speed (s) that minimises the cost.

| Step | Instruction | Working |
| --- | --- | --- |
| 1. | Differentiate the cost function with respect to s. | dCds = 4s − 24 |
| 2. | Set the derivative to zero to find the minimum point. | 4s − 24 = 0 |
| 3. | Solve for s (the optimal speed). | 4s = 24 s = 6 |
| 4. | Calculate the minimum cost (Substitute s back into C(s)) | C(6) = 2(6)² − 24(6) + 150 = 72 – 144 + 150 = £78 |

# Student activity – Differentiation for engineering optimisation

You are a Junior Design Engineer tasked with reducing material waste and increasing the efficiency of a key component. You must mathematically prove the optimal dimensions for a product casing using a fixed amount of material and the speed of a machine which produces this casing

In manufacturing, material costs and waste disposal are significant expenses. In this example, the company is producing a protective casing for a new electronic control unit. Standard aluminium stock sheets are supplied at 30 cm × 20 cm.

You must use differentiation to find the dimensions that result in the maximum possible volume for an open-top box made from a 30 cm × 20 cm sheet of material with box height x.

Any mathematical solution where the total cut width exceeds the sheet dimensions (i.e., 2x≥20 cm) is physically impossible and must be rejected.

The volume equation for this box is: V(x)=(30-2x)(20-2x)x.

Determine the value of x (the cut-out square side length, which is also the height of the folded box) that maximises the volume.

| Step | Instruction | Calculation |
| --- | --- | --- |
| 1. | Expand the equation to the form V(x)=ax3+bx2+cx. | V(x) = 4x3 − 100x2 + 600x |
| 2. | Differentiate to find the gradient function dVdx | dVdx = 12x2 − 200x + 600 |
| 3. | Set dVdx = 0 and simplify the resulting quadratic equation by dividing by 4. | 12x2-200x+600=0 12x2-200x+6004=0 3x2-50x+150=0 |
| 4. | Solve for x using the Quadratic Formula: x=-b±b2-4ac2a. | x1 = 3.923 x2 =12.744 |
| 5. | Validate x. Only one value is physically possible (must be less than 10 cm). State the optimal x. | Optimal x = 3.92 cm |
| 6. | Calculate the Maximum Volume Vmax (Substitute the optimal x back into the original V(x) equation from Step 1). | Vmax = 4(3.92)3 - 1003.922 + 600(3.92)= 1056.31 cm3 |

Differentiation practice question answers

| Problem | dydx or dVdx | Calculation for final answer | Final answer |
| --- | --- | --- | --- |
| Find the gradient of y=5x3-2x2+7x-1 when x=3. | 15x2− 4x + 7 | 15(3) ² − 4(3) + 7 = 130 | 130 |
| The displacement in metres (s) of a remote-controlled device is s(t)=2t3-5t2+4t. Find the velocity at t=2seconds. | v(t) = 6t² − 10t + 4 | 6(2)² − 10(2) + 4 = 8 | 8 m/s |
| A liquid's pressure (P) changes with temperature (T) in ºC by P(T)=0.5T2-8T+30. Find the temperature (T) at which the pressure is a minimum. | dPdT = T − 8 | T-8=0 | T=8°C It is a minimum because d2PdT2=1 and is positive. |
| The velocity (v) of a train in m/s is v(t)=4t2+2t. Find the acceleration at t=10 seconds. | a(t) = 8t + 2 | 8(10) + 2 = 82 | 82 m/s² |