Integration Worksheet

# Worked example 1 – Definite integral (cumulative change)

This example shows how a definite integral (limits) calculates a total quantity, such as volume over time (flow rate).

HINT: Definite Integral: Used to find the total quantity between two points

Problem

The flow rate (Q) of water through a cooling pipe in m3/min is given by the function:

Q(t)=3t2+2t+5

Find the total volume of water that passes through the pipe during the first 5 minutes (t=0 to t=5).

HINT: When integrating, add 1 to the power and divide by the new power.

| Step | Instruction | Working |
| --- | --- | --- |
| 1. | Formulate the integral. Total volume, V is the integral of the flow rate function, Q(t). |  |
| 2. | Integrate term by term. (∫axndx=an+1xn+1) |  |
| 3. | Apply the limits (definite integral). Substitute the upper limit (t=5) then the lower limit (t=0) and subtract. |  |
| 4. | Calculate the result. |  |

# Worked example 2 – Indefinite integral

HINT: Indefinite integral: Used to find the general formula

Problem

The velocity (v) of a component starting from a test rig is given by:

v(t)=4t3-6t+2 (m/s).

If the component starts from a position s=3 m (when t=0), find the equation for its displacement s(t).

| Step | Instruction | Working |
| --- | --- | --- |
| 1. | Integrate the velocity function to find the displacement function s(t). |  |
| 2. | Use the initial condition to solve for C. (When t=0, s=3). |  |
| 3. | Write the final, specific equation for displacement. |  |

HINT: Always add the constant C because integration tells us the shape of the movement, but C represents the specific starting position which cannot be known from the velocity alone.

# Student activity – Integration for cumulative effects

You are a Maintenance Engineer responsible for tracking the performance and energy usage of manufacturing machinery. You cannot simply read the total energy meter because the machine's power output fluctuates over time.

You must use the mathematical technique of integration to calculate the exact total energy consumed by the machine over a specific period, given the function that describes its instantaneous power output.

The instantaneous power consumption, P (in Watts) of a new motor is modelled by the function: P(t)=6t2+4t+5, where t is the time in seconds.

Calculate the total energy, E (in Joules) consumed by the motor over the first 5 seconds of operation (t=0 to t=5). Remember: Energy is the integral of power over time: E=∫P(t)dt. The first step has been completed for you.

| Step | Instruction | Calculation |
| --- | --- | --- |
| 1. | Formulate the definite integral. (limits t=0 and t=5). | E= 05(6t² + 4t + 5)dt |
| 2. | Integrate the power function P(t) term by term. | E= 05 |
| 3. | Apply the upper limit (t=5). | Eupper= |
| 4. | Apply the lower limit (t=0). | Elower= |
| 5. | Calculate the total energy (Eupper-Elower). | total energy, E= joules (J) |

Integration practice questions

| Problem | Integrated expression | Calculation for final answer | Final answer |
| --- | --- | --- | --- |
| Find the integral ∫(9x2-4x+1)dx. |  |  |  |
| Velocity → Displacement (indefinite) The velocity of a vehicle is vt=3t2+2. If the initial displacement s0=10 m Find the equation for displacement s(t). |  |  |  |
| Force → Impulse (definite) The force (F, in N) applied to a component is F(t)=5t-3. Calculate the total Impulse (area under the curve) generated from t=1 to t=4 seconds. |  |  |  |
| Calculate the definite integral: 13x2+x dx. |  |  |  |