Formulae and rearrangement answers

# Worked example 1: Using formulae

The pitch control arm has a pivot radius of 1.2 m.

The scale model exerts a weight (force) of 2000 N at this distance.

Calculate the required torque in Nm.

Use the formula:

T= F × r

where

T = torque

F = force

r = radius

Answer

T=F×r

T=2000×1.2=2400 Nm

# Worked example 2: Rearranging formulae

The pitch control arm requires a maximum torque of 3000 Nm for safety.

The pivot radius is 1.5 m.

Calculate the maximum force that can be applied at this distance.

Use the formula above.

Answer

T=F×r

⟹F=Tr

F=3000 1.5 =2000 N

## Formula application in a pitch control actuator system

For each calculation:

- write the formula

- identify the unknown and rearrange the formula if required

- substitute known values

- calculate (treat standard form calculations with care and convert to the correct units if necessary).

| Engineering parameter | Calculation | Calculated value |
| --- | --- | --- |
| Torque (Mechanical) | Τ = F × r=2500 ×1.5 | 3750 Nm |
| Piston Area (Pascal's Law) | Rearrange the formula: P =FA →A=FP Substitute values and calculate: A =25001.2×107 =2.083×10-4 m2 | 2.083×10-4 m2 |
| Piston Diameter (Geometry) | Rearrange the formula: A=πr2→r=Aπ Substitute values and calculate radius: r=Aπ=2.083×10-4π= 0.00814 m Substitute values and calculate diameter: d=2r=2×0.00814 =0.016286 m Convert from m to mm: = 0.01628 m×1000 mm1 m =16.28 mm | 16.28 mm |
| Current (Ohm's Law) | Rearrange the formula: V=I×R→I=VR Substitute values and calculate: I=VR= 400 8=50 A | 50 A |
| Accumulator Volume (Boyle's Law) | Rearrange the formula: P1​V1 ​= P2​V2 →V2=P1​V1P2 Substitute values and calculate: V2=P1​V1P2=(1×105 )×(0.05)1.2×107 =4.167×10-4 m3 | 4.167×10-4 m3 |

# Practice questions

- The pitch control arm has a pivot radius (r) of 0.8 m. A force (F) of 3200 N is applied. What is the resulting Torque (T) in Nm?

Τ = F × r=3200 ×0.8 =2560 Nm

- A hydraulic piston has an area (A) of 5×10-4 m2. If it needs to exert a force (F) of 5000 N, what is the required pressure (P) in Pa?

P =FA=50005×10-4 =50000.0005=10 000 000=1×107Pa

- A component in the circuit is drawing a current (I) of 25 A when connected to a voltage (V) of 400 V. What is the resistance (R) of the component in Ω?

V=I×R

→R=VI		R=VI= 400 25 =16 Ω