Formulae and rearrangement Worksheet

# Worked example 1: Using formulae

The pitch control arm has a pivot radius of 1.2 m.

The scale model exerts a weight (force) of 2000 N at this distance.

Calculate the required torque in Nm.

Use the formula:

T= F × r

where

T = torque

F = force

r = radius

# Worked example 2: Rearranging formulae

The pitch control arm requires a maximum torque of 3000 Nm for safety.

The pivot radius is 1.5 m.

Calculate the maximum force that can be applied at this distance.

Use the formula above.

## Formula application in a pitch control actuator system

You are a Level 4 Engineering Design and Development Technician Apprentice in an automotive racing team’s Aerodynamics department. Your current task is to finalise the specifications for a new Hydraulic Pitch Control Actuator System. This system will precisely adjust the angle of the scale model car in the wind tunnel.

A single calculation error, especially in formula arrangement or unit conversion, could lead to severe testing delays, component failure, or invalidate millions of pounds worth of test data. Your role is to calculate the crucial parameters of the system.

# Calculating system parameters

Complete the calculations in the table by applying and rearranging the correct formulae for the Hydraulic Pitch Control System. 
Then complete the questions that follow.

| Engineering parameter | Known values | Unknown quantity | Formula | Calculated value |
| --- | --- | --- | --- | --- |
| Torque (Mechanical) | force, F = 2500 N radius, r =1.5 m | torque, T (Nm) | Τ = F × r |  |
| Piston Area (Pascal's Law) | force, F Use force from previous calculation. pressure, P = 1.2×107 Pa | area, A (m2) | P =FA Rearrange for A. |  |
| Piston Diameter (Geometry) | area Use area from previous calculation. | diameter, d (mm) | A = πr2 d=2r Rearrange for radius r then find diameter d. |  |
| Current (Ohm's Law) | voltage, V = 400 V resistance, R = 8 Ω | current, I (A) | V = I×R Rearrange for I. |  |
| Accumulator Volume (Boyle's Law) | initial pressure, P1​ = 1×105 Pa initial volume, V1=0.05 m3 final pressure, P2​ = 1.2×107 Pa | final volume, V2​ (m3) | P1​V1 ​= P2​V2 Rearrange for V2​. |  |

# Practice questions

- The pitch control arm has a pivot radius (r) of 0.8 m. A force (F) of 3200 N is applied. What is the resulting Torque (T) in Nm?

- A hydraulic piston has an area (A) of 5×10-4 m2. If it needs to exert a force (F) of 5000 N, what is the required pressure (P) in Pa?

- A component in the circuit is drawing a current (I) of 25 A when connected to a voltage (V) of 400 V. What is the resistance (R) of the component in Ω?