Calculating key parameters Worksheet

# Belt system commissioning calculations

You are a Commissioning Engineer at a manufacturing plant. A new conveyor system, driven by a series of belt and pulley systems, has just been installed.

The mechanical contractor who installed the system provided a data sheet of theoretical performance values, but they used mixed units. Your critical task is to review and confirm the key operational parameters – speed, torque, and power – to ensure the system runs safely and efficiently before final handover.

## Calculating key parameters

| Engineering parameter | Given values | Required unit | Conversion factor/ formula | Final answer |
| --- | --- | --- | --- | --- |
| Belt speed | D1=150 mm N1=1500 rpm | v (m/s) | v=πD1N160 |  |
| Speed ratio (belt system) | N1=1500 rpm N2=500 rpm | ratio | Speed ratio = N1N2 |  |
| Driven pulley diameter | D1=150 mm Speed ratio from previous row | D2 (mm) | Speed ratio, D2D1=N1N2 |  |
| Driving torque (pulley) | Pin=7.5 kW N1=1500 rpm | T1 (Nm) | P=2πNT60 |  |
| Output power (efficiency) | power input, Pin=7.5 kW η, efficiency=92% | Pout (kW) | η=PoutPin×100% |  |

Questions

- The driving motor (primary pulley) is designed to operate with a torque (T1) of 55 Nm. The pulley rotates at a speed (N1) of 1800 rpm.

Calculate the required power input (Pin) to the motor in kilowatts (kW).

- The belt system has a primary pulley diameter (D1) of 100 mm and a secondary (driven) pulley diameter (D2) of 350 mm. The primary pulley is driven by a motor running at 2100 rpm (N1).

Calculate the rotational speed of the driven machine (secondary pulley speed, N2) in revolutions per minute (rpm).

- A belt drive system must deliver an output power (Pout) of 5.8 kW to the driven machine. The motor supplying the power has an input power (Pin) rating of 6.5 kW.

Calculate the minimum operational efficiency (η) the belt system must achieve, expressing your answer as a percentage (%).

- The drive belt is operating at a linear belt speed (v) of 15 m/s. The motor connected to the primary pulley runs at a speed (N1) of 1200 rpm.

Calculate the required diameter (D1) of the primary pulley in millimetres (mm).