Calculating key parameters Worksheet answers

# Belt system commissioning calculations

You are a Commissioning Engineer at a manufacturing plant. A new conveyor system, driven by a series of belt and pulley systems, has just been installed.

The mechanical contractor who installed the system provided a data sheet of theoretical performance values, but they used mixed units. Your critical task is to review and confirm the key operational parameters – speed, torque, and power – to ensure the system runs safely and efficiently before final handover.

| Engineering parameter | Calculation | Final SI Answer |
| --- | --- | --- |
| Belt speed | Convert diameter to m: 150 mm=150 mm ×1 m1000 mm=0.150 m Substitute values and calculate: v=πD1N160=π×0.150 ×1500 60 =11.78 m/s | 11.78 m/s |
| Speed ratio | Speed ratio = N1N2=1500 500 =3 | 3 (or 3 :1) |
| Driven pulley diameter | Rearrange formula: D2D1=N1N2→D2=N1N2×D1 Substitute values and calculate, remembering to use the speed ratio from the previous row: D2=N1N2×D1 =3×150 mm=450 mm | 450 mm |
| Driving torque | Rearrange formula: P=2πNT60→T=60P2πN Substitute values and calculate, remembering to convert 7.5 kW to 7500 W: T1=60P2πN=60×75002π×1500=47.8 Nm | 47.8 Nm |
| Output power | Rearrange formula: efficiency=PoutPin →Pout=efficiency ×Pin Substitute values and calculate: Pout=efficiency ×Pin=0.92 ×7.5=6.9 kW | 6.9 kW |

Questions

- The driving motor (primary pulley) is designed to operate with a torque (T1) of 55 Nm. The pulley rotates at a speed (N1) of 1800 rpm.

Calculate the required power input (Pin) to the motor in kilowatts (kW).

Find the formula that relates torque and power using rpm:

P=2πNT60

Substitute known values and calculate:

P=2πNT60=2π×1800×5560=10 367 W

Convert W to kW:

10 367 W×11000 =10.367 kW

The required input power is 10.367 kW.

- The belt system has a primary pulley diameter (D1) of 100 mm and a secondary (driven) pulley diameter (D2) of 350 mm. The primary pulley is driven by a motor running at 2100 rpm (N1).

Calculate the rotational speed of the driven machine (secondary pulley speed, N2) in revolutions per minute (rpm).

Find the formula that relates the speed ratios:

D2D1=N1N2

Rearrange to make N2 the subject:

D2D1=N1N2→ D1D2=N2N1→D1D2×N1=N2N1×N1→ D1N1D2=N2→ N2=D1N1D2

Substitute known values:

N2=D1N1D2=100×2100350=600 rpm

Note that because both diameters are in mm and both angular speeds are in rpm, there is no need to convert the SI units first because the mm units cancel, but it is always safe to convert to SI units first.

The rotational speed of the driven machine is 600 rpm.

- A belt drive system must deliver an output power (Pout) of 5.8 kW to the driven machine. The motor supplying the power has an input power (Pin) rating of 6.5 kW.

Calculate the minimum operational efficiency (η) the belt system must achieve, expressing your answer as a percentage (%).

Substitute values into the efficiency formula and calculate:

η=PoutPin×100%=5.86.5×100%

=89.23%

The minimum operational efficiency is approximately 89.23%.

Note that because efficiency is a ratio, if both powers are in the same unit, the calculation will be correct, but it is always safe to convert to SI units first.

- The drive belt is operating at a linear belt speed (v) of 15 m/s. The motor connected to the primary pulley runs at a speed (N1) of 1200 rpm.

Calculate the required diameter (D1) of the primary pulley in millimetres (mm).

Find the formula that relates belt speed to rpm and diameter:

v=πDN60

Rearrange the formula for diameter by multiplying both sides by 60 and dividing both sides by πN.

v=πDN60→D=60vπN

Substitute known values and calculate:

D=60vπN=60×15 π×1200=0.239 m

Convert m to mm:

0.239 m= 0.239 m×1000 mm m=239 mm

The required diameter (D1) of the primary pulley is approximately 239 mm.