Resource 1 Worksheet

1	You need to order vinyl flooring for this renovation. Calculate the internal floor area.

Add 10% for cutting waste and state the total area to order.

(6 marks)

2	Steel I-Beams (Universal Beams) each have a mass of 250 kg.

A hydraulic platform can safely support a load of 50 kN.

(a)	What is the maximum number of beams that the platform can safely support?

(4 marks)

The site safety manager requires a 15% safety margin on the lift capacity.

(b)	Calculate the maximum number of beams you can lift while maintaining this safety margin.

(3 marks)

3	(a)	Define luminous efficacy.

(1 mark)

(b)	 A lamp is rated at 36 W and emits a total luminous flux of 1800 lumens.

Use this formula to calculate the luminous efficacy of the lamp.

luminous efficacy= luminous fluxpower

Include units in your answer.

(3 marks)

(c)	The building specifications require all lighting to have an efficacy of at least
60 lm/W to meet sustainability targets. Does this lamp meet the specification? (Yes/No)

(1 mark)

4	You are ordering ready-mix concrete for a foundation pour.

Concrete has a density of 2400 kg/m3. There are 1000 kg in one tonne.

(a)	Calculate the volume of 5 tonnes of concrete.

(4 marks)

The foundation of a house has dimensions 12 m × 8 m × 0.75 m.

A particular pump can pump concrete at the rate of 96 m3 per hour.

(b)	Work out, to the nearest minute, how long it takes to fill the foundations.

(4 marks)

(c)	Concrete is sold in cubic metres (m3). Based on your volume calculation, how much should you order to the nearest 0.5 m³ to ensure you do not run short?

(2 marks)

5	The thermal expansion of a material is described by the formula:

ΔL = α × L × ΔT

(a) Explain the meaning of α in this formula and describe what is meant by thermal expansion.

(b) A 10 m length of wire increases in length by 15 μm when its temperature rises by 4°C. Calculate the value of α for this wire, giving your answer with units.

(c) Explain why thermal expansion of wire needs to be considered when designing overhead power lines.

(12 marks)