Formulae sheet

The variables are written in format: variable in words, symbol (unit)

| Basic maths | area, A m2= length, l m× width, w m volume of a cylinder or prism, V (m3) = height, h (m) × area of base, A (m2) capacity litres=volume (m3)1000 density, ρ kgm3=mass, m (kg)volume, V (m3) |
| --- | --- |
| Materials | change in length, ∆L m= α mmK× ∆T K× L m where α is the coefficient of thermal expansion ΔT is temperature change L is original length |
| Mechanics | weight, W (N) = mass, m (kg) × gravitational constant, g (m/s2) force, F (N) = mass, m (kg) × acceleration, a (m/s2) work done, W (J) = force, F (N) × distance d (m) energy transferred, E (J) = force, F (N) × distance d (m) moment, M Nm= force, F N× perp. distance, d (m) torque, T Nm= force F N× perp. distance, d (m) power, P W=work (energy), W (J)time, t (s) pressure, p pa orNm2=force, F (N)area, A (m2) |
| Light | luminous efficacy, η lmW= luminous flux,φ (lm) power, P (W) |

| Electricity | voltage, V (V) = current, I (A) × resistance, R (Ω) power, P (W)= voltage, V (V) × current, I (A) power, P (W)= current2, I2 (A2) × resistance, R (Ω) total resistance in parallel in ohms (Ω): 1Rtotal=1R1+1R2+ 1R3+… total resistance in series in ohms (Ω): Rtotal=R1+R2+R3+… transformers VsVp=NsNP, IsIP=NpNs , VPIP=VsIs where Vs, VP voltage in secondary and primary coils, V Is, IP current in secondary and primary coils, A Ns, NP number of turns in secondary and primary coils |
| --- | --- |
| Heat | heat energy, Q J= m kg× c Jkg K× ΔT K where m is mass c is specific heat capacity ΔT is temperature change thermal resistance, r m2KW=thickness, t (m)thermal conductivity, k (Wm K) heat transfer coeff., U (Wm2K )=1thermal resistance, R (m2KW) heat loss, Q W=UWm2K×A m2× ΔT K where U is the heat transfer coefficient A is area ΔT is temperature difference power, P W=energy, Q (J)time, t (s) |

Long form of units referred to in formulae

| Base SI units |  |
| --- | --- |
| Unit | Shorthand |
| kilogram | kg |
| second | s |
| metre | m |
| ampere | A |
| kelvin | K |
| candela | cd |

| Derived SI units |  |
| --- | --- |
| Unit | Shorthand |
| newton | N |
| pascals | Pa |
| joules | J |
| watt | W |
| volts | V |
| ohms | Ω |
| lumens | lm |
| hertz | Hz |
| ohm metres | Ωm |

Metric prefixes

| Common metric prefixes |  |
| --- | --- |
| Prefix (symbol) | Value in standard form |
| giga (G) | 1 × 10⁹ |
| mega (M) | 1 × 10⁶ |
| kilo (k) | 1 × 10³ |
| centi (c) | 1 × 10⁻² |
| milli (m) | 1 × 10⁻³ |
| micro (µ) | 1 × 10⁻⁶ |
| nano (n) | 1 × 10⁻⁹ |