Circuit calculations: Worksheet answers

1. Voltage drop

In a commercial installation, failing to account for ‘voltage drop’ can lead to system failure.
You have a 9 V battery and want to power two red LEDs in a single series string.
Each LED has a forward voltage (Vforward) of 2.2 V.
Explain whether the LEDS will light.

The total forward voltage of the LEDs is:

2.2 V + 2.2 V = 4.4 V.

The 9 V battery is sufficient because:

9 V > 4.4.

So, the LEDs will light because the total forward voltage (4.4 V) is less than the supply voltage (9 V).

Note: The remaining 4.6 V should be dropped across a resistor to limit the current and prevent the LEDs from burning out.

2. Resistor sizing

You have a 9 V supply and a blue LED (Vforward=3.2 V).
You want a current of 0.02 A (20 mA).
Calculate the required resistor value using:

R=(Vsupply-Vforward)Itarget

Step 1: Substitute the known values into the equation.

R=(9.0-3.2)0.02

Step 2: Calculate the resistor value.

R=5.80.02

R=290 Ω

Note: The voltage difference between the supply voltage and the LED forward voltage (9.0 V-3.2 V=5.8 V) is dropped across the resistor. In practice, the nearest higher standard resistor value, such as 300 Ω (or  330 Ω), would be selected to ensure the current does not exceed the target value of 20 mA.

3. Transistor gain

You measure a base current (IB) of 0.005 A (5mA) and a collector current (IC) of 
0.15 A (150 mA). Calculate the gain (β) using:

β=ICIB

Step 1: Substitute the known values into the equation and calculate the gain.

β=0.15 A0.005 A

β=300

Context: This means the collector current is 300 times the base current. This is a typical gain value for a BC547 transistor.

Extension question

4. Voltage divider

You have a 5 V supply and want to bias the base of an NPN transistor at ~0.7 V using a voltage divider.
The resistor R2 has a value of 4.7 k.
Calculate the value of the second fixed resistor R1 using:

Vout=Vin×R2R1+R2

Step 1: Substitute the known values.

0.7=5×4700R1+4700

Step 2: Rearrange to make R1 the subject.

- Divide both sides by 5:

0.14=4700R1+4700

- Multiply both sides by R14700:

0.14×(R1 +700)=4700

- Expand the bracket:

0.14 R1+658=4700

- Subtract 658 from both sides:

0.14 R1=4042

- Divide by 0.14:

R1=40420.14=28 871 Ω

R1≈28.9 kΩ

Step 3: Choose practical resistor values (pick standard values that match the ratio).

- R1=30 kΩ

- R2=4.7 kΩ

Step 4: Check the output.

- Vout=5×4.730+4.7=5×4.734.7≈0.68 V

Result:

- Output ≈ 0.68 V (close enough for turning on a silicon transistor)

- Slight variation is fine because base-emitter voltage is not exactly 0.7 V in reality.