Circuit calculations: Worksheet

1. Voltage drop

In a commercial installation, failing to account for ‘voltage drop’ can lead to system failure.
You have a 9 V battery and want to power two red LEDs in a single series string.
Each LED has a forward voltage (Vforward) of 2.2 V.
Explain whether the LEDS will light.

2. Resistor sizing

You have a 9 V supply and a blue LED (Vforward=3.2 V).
You want a current of 0.02 A (20 mA).
Calculate the required resistor value using:

R=(Vsupply-Vforward)Itarget

3. Transistor gain

You measure a base current (IB) of 0.005 A and a collector current (IC) of 0.15 A.
Calculate the gain (β) using:

β=ICIB

Extension question

4. Voltage divider

You have a 5 V supply and want to bias the base of an NPN transistor at ~0.7 V using a voltage divider.
The resistor R2 has a value of 4.7 k.
Calculate the value of the second fixed resistor R1 using:

Vout=Vin×R2R1+R2