Glossary

| Term | Definition |
| --- | --- |
| Active region | The operating condition of a transistor between fully off and fully on. In the dimmer circuit, voltage measurements help confirm whether the transistor is off, on, or somewhere in between. |
| Alternating current (AC) | An electrical supply in which the direction of current changes periodically. The teaching materials contrast AC with direct current when recapping prior electrical knowledge. |
| Ambience | The desired lighting atmosphere in the cinema-room task. The LED dimmer must vary brightness smoothly to help create a comfortable lighting environment. |
| Ammeter | A measuring instrument used to measure current. In the practical task, current measurements are needed to check base current and collector current. |
| Anode | The positive side of an LED or diode. In the practical pre-power check, the LED anode is identified by the long leg and should be connected to the positive side. |
| Base | The transistor terminal that acts as the gate or trigger. A small current applied to the base allows a larger current to flow between the collector and emitter. |
| Base current | The small control current flowing into the transistor base from the control circuit or potentiometer. It is measured when calculating transistor gain. |
| Base resistor | A resistor used to limit the current entering the transistor base so that the transistor is not damaged. |
| Battery | A low-voltage power source used to supply the circuits in the teaching activities, typically within the 5–9 V range for the LED dimmer circuit. |
| Bill of materials (BOM) | A list of the components required to assemble the circuit. In the LED dimmer task it includes the power supply, potentiometer, transistor, resistors, LED and breadboard. |
| Breadboard | A prototyping board used to assemble and test an electronic circuit without making a permanent circuit board. |
| CAD software | Computer-aided design software used to produce a circuit diagram or layout. The materials refer to using pencil or CAD software when drawing the simulated circuit. |
| Cathode | The negative side of an LED or diode. Checking the cathode helps ensure that the component is connected with the correct polarity. |
| Circuit diagram | A drawing that represents an electrical or electronic circuit using standard electronic symbols and labels for components, supply voltage, resistor values and transistor terminals. |
| Collector | The transistor terminal where the larger controlled current is present. In the LED dimmer, the LED is connected in the collector circuit. |
| Collector current | The current flowing through the load or LED side of the transistor circuit. It is measured and compared with base current to calculate transistor gain. |
| Component orientation | The correct physical placement and direction of components before power is applied. The teaching materials highlight LED polarity and transistor pinout checks to avoid damage. |
| Current | The flow of electrical charge around a circuit. In a series circuit it is constant at all points; in a parallel circuit it splits between branches. |
| Current control | The use of a correctly calculated resistor to keep LED and transistor current below the specified safe limit, under 20 mA in the LED dimmer task. |
| Current-limiting resistor | A resistor placed in series with an LED to restrict current to a safe operating value and prevent the LED from overheating or burning out. |
| Cut-off | The off state of the transistor. In the teaching materials, the BC547 remains cut off when the base voltage is below the value needed to open the transistor gate. |
| DC power supply | A direct-current source used to power the circuit. The LED dimmer activity uses a 5–9 V DC supply or battery. |
| Dimmer circuit | An LED circuit designed so that brightness can be varied smoothly. In the practical project, the potentiometer controls the transistor so that the LED can be dimmed. |
| Diode | An electronic component that allows current to flow in one direction while blocking current in the opposite direction. |
| Direct current (DC) | Current that flows in one direction. The teaching activities use DC circuits powered by a battery or low-voltage power supply. |
| Electrical circuit | A connected arrangement of components and a power source that allows current to flow and electrical quantities such as voltage, current and resistance to be measured or calculated. |
| Electronics | The use of electricity to make things happen or provide control, rather than simply supplying electrical power. The teaching materials use LED dimming and transistor switching as examples. |
| Emitter | The transistor terminal where current exits to ground in the NPN transistor circuit. |
| Forward voltage | The voltage needed across an LED or diode when it is conducting in the forward direction. It is used to calculate the series resistor for LED protection. |
| Gain | The ratio showing how much larger the collector current is than the base current in a transistor. In the materials it is calculated as collector current divided by base current. |
| Ground | The 0 V reference point in the circuit. In the transistor circuit, the emitter is expected to be connected to ground. |
| LED | A light-emitting diode that transfers electrical energy into light. Its polarity and current must be controlled so that it operates safely. |
| LED current-limiting resistor | The resistor connected with the LED to keep the LED current within a safe value. It is calculated using the supply voltage, LED forward voltage and desired LED current. |
| Multimeter | A measuring instrument used to measure electrical quantities such as voltage and current during testing and diagnostic checks. |
| NPN transistor | A transistor type used in the teaching circuits as an electronic switch or current amplifier. The BC547 or equivalent is used in the dimmer circuit. |
| Ohm’s law | The relationship between voltage, current and resistance used in circuit calculations. The video brief identifies V = IR as a common workplace formula. |
| Parallel circuit | A circuit in which current splits at a junction into separate branches. Each branch receives the supply voltage. |
| Pinout | The order and function of a component’s terminals. The materials require students to verify the transistor pinout, labelled CBE, before applying power. |
| Polarity | The positive and negative orientation of a component or supply connection. Correct LED polarity and red/black power connections are highlighted as safety checks. |
| Potentiometer | A variable resistor with three terminals, one of which is movable. It provides a varying resistance or voltage output and is used to control the transistor base current in the LED dimmer. |
| Power | The rate at which electrical energy is transferred in a circuit. The teaching materials identify P = VI as a common calculation used in the workplace. |
| Power dissipated | The electrical power converted into heat by a component or resistor combination. It is checked to ensure the circuit remains within safe power limits. |
| Power safety check | A calculation used to confirm that a resistor’s power dissipation is below the safe limit, such as 0.25 W in the LED protection task. |
| Power supply | The part of the circuit that provides electrical energy to drive the components. |
| Prototype | A trial version of the circuit created before the physical build. Students use Circuit Wizard or Tinkercad® to prototype the LED dimmer circuit. |
| Resistance | The opposition to current flow in a circuit. It is used in calculations for series circuits, parallel circuits and LED resistor sizing. |
| Resistor | A component used to limit current or create a required voltage drop in a circuit. |
| Series circuit | A circuit in which current must pass through all components in a single path. Total resistance is the sum of the individual resistances. |
| Series resistor | A resistor connected in series with an LED so that the remaining voltage is dropped across the resistor and the LED current is limited. |
| Short circuit | An unsafe direct connection between the positive and negative supply rails. The practical task requires students to ensure the rails never come into direct contact. |
| Simulation | A digital prototype of the circuit used to test how the design behaves before building it physically, using tools such as Circuit Wizard or Tinkercad®. |
| Supply rails | The positive and negative lines on the breadboard that distribute the power supply around the circuit. |
| Supply voltage | The voltage provided by the power source, such as a 5–9 V battery or DC power supply in the LED dimmer task. |
| Thermal damage | Damage caused when components become too hot. The teaching materials link this risk to excessive current, incorrect orientation or transistor breakdown. |
| Transistor | A semiconductor device used to switch electronic signals or amplify them. A small current at the base controls a larger current between collector and emitter. |
| Transistor gain (hFE) | The current gain of a transistor, calculated as collector current divided by base current. It is dimensionless and is compared with the manufacturer’s data sheet. |
| Variable resistor | A resistor whose resistance can be changed. In the teaching materials, the potentiometer acts as a variable resistor. |
| Voltage | The electrical potential difference that drives current through a circuit. In a series circuit the supply voltage is divided between components; in a parallel circuit each branch receives the supply voltage. |
| Voltage checkpoints | Diagnostic voltage measurements taken at the base, collector and emitter to check whether the transistor is off, on or in its active region. |
| Voltage divider | A pair of resistors used to provide a chosen output voltage from a supply voltage, such as biasing the base of an NPN transistor at about 0.7 V. |
| Voltage drop | The portion of the supply voltage used across a component or resistor. The circuit calculations use voltage drop to decide whether LEDs will light and what resistor is needed. |
| Voltmeter | A measuring instrument used to measure voltage at points in the circuit, such as the base, collector and emitter voltage checkpoints. |