Data sheet

| Typical LED working voltage |  |  |
| --- | --- | --- |
| LED colour | Forward voltage (Vf) | Typical current (If) |
| Red | 1.8–2.1 V | 10–20 mA |
| Amber | 2.0–2.2 V | 10–20 mA |
| Orange/Yellow | 1.9–2.2 V | 10–20 mA |
| Green | 2.0–3.1 V | 10–20 mA |
| White/Blue | 3.0–3.4 V | 20 mA |

Series resistor for LED (𝑅𝑠𝑒𝑟𝑖𝑒𝑠)

To prevent an LED from burning out, use the following to calculate the required resistor:

R=Vs-VfIf

where,

Vs is the supply voltage (e.g., 9 V battery)

Vf is the forward voltage of the LED (from the table above)

If is the desired LED current (e.g., 0.02 A for 20 mA).

Transistor gain (hFE)

The ratio of collector current to base current:

where,

hFE is the current gain (dimensionless)

Ic is the collector current (flowing through the load/LED)

IB is the base current (flowing from the control/potentiometer).