Glossary

| Term | Definition |
| --- | --- |
| Actuation | The process of causing a component or mechanism to move. In the teaching materials, actuation occurs when the brake pedal, trigger or valve action starts movement in the hydraulic or pneumatic system. |
| Actuation input | The initial action that starts the system. Examples include pressing the brake pedal in the hydraulic braking system or pulling the trigger on the pneumatic nail gun. |
| Actuator | A component that converts fluid or air pressure into mechanical movement. In the materials, cylinders and pistons act as actuators by producing movement from pressure. |
| Air inlet / coupler | The connection point where compressed air enters the pneumatic nail gun from the air source. |
| Augmented reality (AR) | A digital viewing method used in the resources to let students inspect 3D models of the hydraulic braking system and pneumatic nail gun. |
| Barrel / nail channel | The front section of the pneumatic nail gun through which the fastener is propelled. |
| Brake booster / servo | A braking system component that uses engine vacuum to increase the force applied by the push rod, reducing the effort required from the driver. |
| Brake disc | A metal disc attached to the wheel hub. It provides the surface that the brake pads clamp onto to slow the vehicle. |
| Brake fluid | An incompressible fluid used in the hydraulic braking system. It transfers pressure through the system and can withstand high temperatures and pressures without boiling. |
| Brake lines and hoses | Parts of the hydraulic braking system that transport brake fluid from the master cylinder to the calipers, transferring hydraulic pressure. |
| Brake pads | High-friction components that press against the brake disc and convert kinetic energy into thermal energy, slowing the vehicle. |
| Brake pedal | The input component pressed by the driver. It applies mechanical force into the braking system. |
| Caliper | A braking component that houses the pistons and holds the brake pads over the brake disc. |
| Compressor | A component used in pneumatic systems to supply compressed air. |
| Cylinder | A chamber in which pressure acts on a piston to create movement. Cylinders are used in both hydraulic and pneumatic systems. |
| Cylinder bore | The internal diameter of a cylinder. In the design task, bore size affects the force that an actuator can produce. |
| Cylinder housing | The part of the pneumatic nail gun that encloses the piston and creates the chamber where pressurised air acts on internal parts. |
| Directional control valve (DCV) | A valve used to control the direction of air or fluid flow. In the nail gun, the trigger opens a 3/2-way directional control valve. |
| Double-acting pneumatic piston | A piston arrangement represented in the pneumatic nail gun schematic as a drive cylinder. It is associated with controlled forward and return movement using air pressure. |
| Exhaust port / outlet port | The path through which used air leaves a pneumatic system. In the nail gun, releasing the trigger vents air to the exhaust port, labelled R/A. |
| Fastener | The nail held in the nail gun magazine and driven into material during final actuation. |
| Feeder mechanism | A guided spring mechanism that keeps fasteners pressed forward in the magazine so they remain aligned. |
| Flow control valve | A valve associated with controlling flow. The materials identify restricted or partially closed flow control as a possible cause of slow actuator movement. |
| Flow direction | The direction in which fluid or air travels through a system. In diagrams, arrows are used to show the flow path when the system is actuated. |
| Flow path | The route taken by the working fluid or compressed air through the system. Students are asked to map this route in their engineering diagrams. |
| Flow rate | The amount of fluid or air delivered over time. In the materials, speed is governed by flow rate rather than pressure. |
| Force output | The mechanical force produced by a hydraulic or pneumatic actuator. Hydraulic systems are associated with higher force output, while pneumatic systems are generally lower force. |
| Hydraulic braking system | A system that uses incompressible brake fluid to transmit pressure from the master cylinder to caliper pistons, causing brake pads to clamp onto brake discs. |
| Hydraulic pressure | Pressure generated in a hydraulic fluid. In the braking example, force applied to the master cylinder creates pressure in the brake lines. |
| Hydraulic system | A fluid power system that uses a liquid working fluid. In the materials, hydraulic systems are associated with high force, high pressure, precise control and good load holding. |
| Hydraulics | The use of liquid fluid power to transmit force and operate mechanisms such as braking systems, lifts and presses. |
| Incompressible | A property of liquids such as hydraulic oil or brake fluid, meaning they do not compress significantly. This allows pressure and force to be transmitted effectively. |
| ISO 1219 symbols | Standard fluid power symbols used in the teaching materials to represent hydraulic and pneumatic components in schematic diagrams. |
| Load holding | The ability of a system to hold a load securely. Hydraulic systems are described as having excellent load holding because liquid does not compress like air. |
| Magazine | The channel in a pneumatic nail gun designed to hold nails or fasteners in a strip. |
| Master cylinder | A hydraulic braking component that converts mechanical force from the pedal into hydraulic pressure by pushing brake fluid through the system. |
| Mechanical force | A physical push or pull applied to a component. In the braking system, the driver applies mechanical force to the brake pedal. |
| Pascal’s law | The principle used in the braking calculations: pressure applied to a confined fluid is transmitted through the system and can produce output force at another piston. |
| Piston | A moving component inside a cylinder. In the braking system, hydraulic pressure pushes the caliper pistons outward; in the nail gun, air pressure propels the piston forward. |
| Pneumatics | The use of compressed air to transmit power and operate mechanisms such as air tools and automated systems. |
| Pneumatic nail gun | A tool that uses compressed air to move a piston rapidly forward and drive a fastener into material. |
| Pneumatic system | A fluid power system that uses compressed air as its working fluid. In the materials, pneumatic systems are associated with speed, simplicity, cleanliness and lower force. |
| Port P | The inlet port on the pneumatic valve, where compressed air enters the system. |
| Port R/A | The outlet or exhaust port through which used air vents from the pneumatic system. |
| Pressure | Force applied over an area. In the teaching materials, pressure determines force output, while flow rate determines speed. |
| Pressure relief valve | A component identified in the test answers as limiting maximum force in a hydraulic system. |
| Pump | A component used in hydraulic systems to circulate liquid in a closed loop under pressure. |
| Reservoir | A container for hydraulic fluid. In schematic work, it is associated with the hydraulic system and fluid supply. |
| Safety contact tip / nose / nozzle | A movable front part of the pneumatic nail gun that helps ensure the gun is positioned correctly before firing. |
| Safety lockout | A safety feature on the pneumatic nail gun. Pressing the safety nozzle against the workpiece is required before the trigger can operate. |
| Schematic | A simplified engineering diagram using standard symbols to show system components, connections and flow direction. |
| Single-acting cylinder | A cylinder symbol used in the hydraulic braking system example to represent the caliper actuator piston. |
| Stroke length | The distance a cylinder or piston travels. In the rear-door design task, stroke length affects whether the door opens far enough. |
| Trigger / throttle | The control mechanism on the pneumatic nail gun that connects to valves and linkages and starts air flow when pulled. |
| Valve | A component that controls the direction, speed or release of fluid or air in a system. |
| Valve plunger / main valve | A pneumatic nail gun component that opens when the trigger is pulled, allowing compressed air to flow into the cylinder and power the piston. |
| Working fluid | The fluid used to transmit power in a system. Hydraulic systems use liquid, while pneumatic systems use compressed air. |