Activity 1 Worksheet: Hydraulic car braking system

Independent observation checklist (what to notice):

| Action | Tick when you have observed this action |
| --- | --- |
| Actuation input: Locate the brake pedal and watch how mechanical pedal force moves the linkage into the servo. |  |
| Force transmission: Observe the rod from the servo entering the main cylinder, displacing incompressible hydraulic fluid. |  |
| Fluid flow: Follow the flow of fluid from the cylinder into the thin brake lines (note that thin lines increase pressure because P = F/A). |  |
| Actuator force: Watch the fluid pressure transmit equally through the lines to the calliper assembly. |  |
| Final actuation: Observe the hydraulic pressure pushing the calliper pistons outward, clamping the brake pads onto the rotating brake discs to stop the wheel (as shown by the red arrows). |  |
| Release state: Watch as the foot is removed from the pedal (or release trigger) and the pistons return to their start position. |  |

Notes/questions:

Use this space to jot down any helpful notes or questions you need to ask.

Activity 1 Worksheet: Pneumatic nail gun

Independent observation checklist (what to notice):

| Action | Tick when you have observed this action |
| --- | --- |
| Safety lockout: Observe how pressing the safety nozzle against a workpiece surface is a prerequisite that releases the physical trigger catch. |  |
| Actuation input: Pull the trigger and notice how it opens the 3/2-way directional control valve (Inlet Port P). |  |
| Fluid flow: Watch compressed air from the source enter the chamber, pushing the valve plunger backward. |  |
| Actuator force: Observe the high-pressure air rushing into the main cylinder and propelling the piston rapidly forward. |  |
| Final actuation: Watch the piston drive the fastener/nail into place. |  |
| Venting / return sequence: Watch how releasing the trigger/nozzle vents the used air to the exhaust (Port R/A), and how compressed air in the return chamber pushes the piston back to its default position. |  |

Notes/questions:

Use this space to jot down any helpful notes or questions you need to ask.