Activity 3 Worksheet: Engineering diagrams

Applied science and mathematics – Pascal’s law

A typical vehicle's front brake system has a master cylinder with a piston diameter of d1 = 20 mm, and a wheel calliper assembly with a piston diameter of d2 = 60 mm. The driver applies an input force of F1 = 150 N to the master cylinder.

Question 1

Calculate the cross-sectional area of the master cylinder piston (A1) in m2.

Use: area = r2

(1 mark)

Question 2

Calculate the fluid pressure, P, generated in the brake lines in pascals (Pa) and megapascals (MPa).

Use:

(2 marks)

Question 3

Calculate the cross-sectional area of the calliper piston (A2) in m2.

(1 mark)

Question 4

Calculate the output braking force (F2) exerted by the calliper piston onto the brake disc.

Use: F2 = P × A2

(1 mark)