Activity 3 Worksheet answers: Engineering diagrams

Applied science and mathematics – Pascal's law

A typical vehicle's front brake system has a master cylinder with a piston diameter of d1 = 20 mm, and a wheel calliper assembly with a piston diameter of d2 = 60 mm. The driver applies an input force of F1 = 150 N to the master cylinder.

Question 1

Calculate the cross-sectional area of the master cylinder piston (A1) in m2.

Use: area = r2

(1 mark)

Piston diameter, d1 = 20 mm = 0.02 m.

Piston radius, r1 = 10 mm = 0.01 m.

Area, A1 = r2 =  × 0.012 = 0.000314 = 3.14 × 10−4 m2

Question 2

Calculate the fluid pressure, P, generated in the brake lines in pascals (Pa) and megapascals (MPa)

Use:

(2 marks)

Force, F1 = 150 N

Area, A1 = 3.14 × 10−4 m2

Pressure,  [1 mark]

Convert to MPa:

[1 mark]

Question 3

Calculate the cross-sectional area of the calliper piston (A2) in m2.

(1 mark)

Piston diameter, d2 = 60 mm = 0.060 m

Piston radius, r2 = 30 mm = 0.030 m

Area A2 = r2 =  × 0.0302 = 2.83 × 10−3 m2

Question 4

Calculate the output braking force (F2) exerted by the calliper piston onto the brake disc.

Use: F2 = P × A2

(1 mark)

Pressure, P =

Area, A2 = 2.83 × 10−3 m2

Force F2 =  = 1350 N (1 mark)

Alternative approach:

Area ratio

Force ratio = piston area ratio = 9

Output force, F2 = 9 × 150 = 1350 N