Information sheet – Hydraulics and pneumatics

Hydraulic car braking system

| Description and role of each component |  |
| --- | --- |
| Brake pedal | The driver applies a mechanical force to the pedal by pushing it down. Links directly to the brake booster. |
| Brake booster (servo) | Uses engine vacuum to increase the force applied by the push rod via a valve system. This reduces the effort the driver must make. |
| Master cylinder | Converts the mechanical force from the pedal into hydraulic pressure by pushing brake fluid through the system. |
| Brake lines and hoses | Transports brake fluid from the cylinder through the brake lines, transferring hydraulic pressure to the callipers at the brake disc. |
| Brake fluid | An incompressible fluid which can withstand very high temperatures and pressures without boiling. |
| Calliper | Houses the pistons, and holds the brake pads over the brake disc. |
| Pistons | Hydraulic pressure from the brake fluid pushes on the calliper pistons, causing the brake pads to squeeze against the brake discs. |
| Brake pads | The brake pads are made from high-friction material. They convert kinetic energy to thermal energy, slowing the car. |
| Brake disc | The brake disc is a metal disc attached to the wheel hub. It provides the surface for the brake pads to clamp onto. |

Pneumatic nail gun

| Description and role of each component |  |
| --- | --- |
| Housing | Protects internal components and holds everything together. It is made of durable material. |
| Handle | Secure place to hold the weight of the device. It is shaped for comfort and stability, often with padding that helps to maintain a safe grip. |
| Magazine with nails (fasteners) | Channel designed to hold the nails in a strip. |
| Air inlet (coupler) | Connection point where compressed air from the air source enters the gun. |
| Trigger (Throttle) | Control mechanism that connects to internal valves and linkages. |
| Feeder mechanism | Uses a guided internal spring to keep fasteners pressed forward in the magazine so they remain aligned. |
| Barrel (Nail channel) | Front section where a fastener is propelled from the gun. |
| Safety contact tip/nose or nozzle | Movable front piece that aligns with the internal parts to make sure that the gun is positioned correctly. |
| Cylinder housing | Encloses the piston and creates a chamber where pressurised air interacts with internal parts. The compressed air is directed into this chamber where it exerts a force on the piston. |
| Valve plunger (main valve) | When the trigger is pulled, the valve plunger opens, allowing compressed air to flow into the cylinder, which then powers the piston. It also controls the flow of compressed air, ensuring that the nail is driven into the material with the correct force and speed. |

Comparison Chart – Hydraulics and pneumatics

| Feature | Pneumatics | Hydraulics |
| --- | --- | --- |
| Working fluid | Compressed air (gas) | Hydraulic oil or fluid (liquid) |
| Compressibility | Compressible | Incompressible |
| Typical operating pressure | Low to medium (typically 5–10 bar / 70–145 psi) | High (often 100–300 bar or more) |
| Force output | Low to medium | Very high |
| Speed of operation | Fast response, rapid movement | Slower but more controlled |
| Control precision | Less precise due to air compressibility | Very precise and smooth |
| Load holding ability | Poor – air compresses and leaks | Excellent – load can be held securely |
| System cleanliness | Clean (air exhausts to atmosphere) | Can be messy if leaks occur |
| Safety | Generally safer; lower stored energy | Higher risk due to high pressures |
| Hazards | Sudden releases and flying components | Stored energy, leaks, injection injuries and poor maintenance |
| Noise | Often noisy (exhaust air) | Quieter in operation |
| Efficiency | Lower efficiency | Higher efficiency |
| Leaks | Waste energy and reduce system performance | Reduce efficiency, contaminate systems and create safety risks |
| Maintenance | Simpler, lower cost | More complex, higher cost |
| Environmental impact | Minimal | Risk of oil contamination |
| System complexity | Simple components and layouts | More complex systems |
| Typical components | Compressor, FRL unit, valves, cylinders | Pump, reservoir, valves, cylinders |
| Common applications | Pick-and-place, air tools, packaging, automation | Presses, lifts, excavators, braking systems |