Plenary Worksheet answers: Engineering diagrams

Comparison of hydraulic and pneumatic systems

Based on your observation of the AR models, compare the two systems across key engineering characteristics.

1. Working fluid (liquid or gas):

Hydraulic braking system: liquid (brake oil / mineral oil).

Pneumatic nail gun: gas (compressed air).

2. Compressibility (incompressible or compressible):

Hydraulic braking system: incompressible (does not compress, providing instant force transmission).

Pneumatic nail gun: compressible (gases compress, storing potential energy but introducing slight delay).

3. Force output and pressure level (high force/high pressure or low force/low pressure):

Hydraulic braking system: high force/high pressure (often 100–300 bar or more, suitable for heavy braking).

Pneumatic nail gun: lower force/low-medium pressure (typically 5–10 bar, suitable for driving fasteners).

4. Speed and response time (rapid/instantaneous or controlled/gradual):

Hydraulic braking system: Controlled, gradual and highly precise response.

Pneumatic nail gun: very rapid, high-speed and repetitive action.

5. Cleanliness and leak safety (risk of fluid spill or safe venting of air):

Hydraulic braking system: high risk of oil leaks, messy, can contaminate work environments and require cleanup.

Pneumatic nail gun: clean, safe, leaks vent harmlessly into the atmosphere.