Activity 2 Worksheet: Test questions

Application

1.	A pneumatic cylinder has a piston area of 0.002 m² and operates at 6 bar.
Calculate the output force. 
Is this realistic for lifting a 300 kg load? Why/why not?

(4 marks)

2.	Two cylinders receive the same flow rate:

Cylinder A: 20 mm diameter

Cylinder B: 40 mm diameter

Explain which moves faster and why.

(2 marks)

3.	A hydraulic system is slow even though pressure is correct.
List two possible causes related to flow.

(2 marks)

4.	A student increases system pressure to make a cylinder extend faster, but nothing changes. Explain why.

(2 marks)

5.	A pneumatic system leaks slightly but still operates.
Describe two performance or efficiency impacts.

(2 marks)

Analysis, evaluation and justification

6.	Justify whether pneumatics or hydraulics are more suitable for a machine that must:

- apply 50 kN force;

- move slowly and precisely;

- operate continuously.

(3 marks)

7.	A manufacturer wants to replace a hydraulic press with a pneumatic system for cost reasons. 
Evaluate this decision, referring to force, safety, efficiency and control.

(3 marks)